TOWNSHIP OF HANOVER

CONSTRUCTION DEPARTMENT

## FENCE REQUIREMENTS

______________________________________________________________________________

- Fences enclosing swimming pools MUST comply with NJ 2006 IRC codes.

- Must be minimum of 48” inches high.

- Must have gate(s) that are self-closing & self-latching.

- Access gates MUST open outward away from pool area.

- Release mechanism on gate shall be min. of 54” inches from bottom of gate.

OR: see attached barrier requirements for further details/alternate locations.

- PLEASE READ ATTACHED 2006 IRC BARRIER REQUIREMENTS SECTION AG105

- Finished (good) side of fence MUST face outward to neighbors.

- No fence shall extend past the property line of the property where said fence is located.

- Fences MUST be located a minimum of 2’ feet from any right of way.

- Maximum height of any fence is 6’ feet high.

- Fences in front yards MUST be:

- At least 50% open.

- May NOT exceed 42” inches high.

- CORNER LOTS: no fence, structure or planting over 24” inches in height shall be erected or maintained within 20’ feet of the intersection formed by the projections of the 2 street lines at the corner.

- The following State mandated forms MUST be completed:

- Building Sub Code application

- Construction Permit application

- MUST provide a copy of most recent survey indicating proposed location and type of fence, i.e.: 6’ foot stockade, 48” inch chain link.

___________________________________________________________________________

IF YOU SHOULD HAVE ANY QUESTIONS OR REQUIRE ASSISTANCE, PLEASE CONTACT US AT (973) 428-2462 DURING THE HOURS OF 8:30AM – 4:30PM, MONDAY THROUGH FRIDAY.

FenceHandout2009