Paterson, NJ Fence Codes

§ 319-6Fence specifications; maintenance.

A.

Specifications for the fence required by § 319-5B are as follows:

(1)

The minimum height shall be six (6) feet.

(2)

The fencing material must be of galvanized metal.

(3)

There shall be a minimum of two-inch outside diameter galvanized pipe for all line posts.

(4)

There shall be a minimum of two-and-one-half-inch outside diameter for all terminal posts, gates, ends and corner posts.

(5)

All terminal posts must be set in concrete footings at a minimum of thirty (30) inches into the ground.

(6)

All line posts must be driven a minimum of thirty (30) inches into the ground.

(7)

Fencing material must be of minimum No. 9 gauge galvanized wire with a two-inch mesh.

(8)

Coil spring minimum No. 7 gauge wire shall be used to provide proper tension.

(9)

In no case should barbed wire, wood or similar materials dangerous to children be used.

B.

It shall be the continuing responsibility of the owner or owners to so maintain the fence and protect the property as hereinabove set forth so long as the same shall remain vacant.